COLLEGE OF ENGINEERING & TECHNOLOGY

Department:  Mechanical Engineering (Mechatronics)

Lecturer:  Dr. Mostafa Rostom

Course Title: Machine Design II			Course Code:  ME 357

Exam:  12th Exam					Marks:   20

Date:  6/5/2012 					Time: 1:20 hour

The figure shows a shaft with two concentrated load. Estimate the shaft’s critical speed without the loads, the first critical speed due to the loads, and the critical speed of the combination.

A helical gear has a 17-tooth. The gear has a right-helix angle of 30°, a normal pressure angle of 20°, and a normal module is 5.0 mm. the gear width is 50 mm. Find:

  - The transverse module and transverse pressure angle

  - The normal circular pitch

  - The pitch, base, addendum, and dedendum diameters of gear

The figure is a schematic drawing of a countershaft that supports two V-belt pulleys. The belt tension on the loose side of pulley A is 15 percent of the tension on the tight side. The shaft is machined from steel with Su=550 MPa, Sy=400 MPa and Se=210 MPa. The shaft runs at 50C. The shaft has a fatigue stress-concentration factors kf=1.66 and kfs=1.63. If the fatigue factor of safety is 1.7, determine the minimum allowable diameter of the shaft at pulley A. The countershaft runs at 1200 rev/min and the bearings are to have a life of 60 kh at a combined reliability of 0.99. Select equal diameter Self Align Ball bearings for use at O and E. the bearing bore range is from 30 to 55 mm.

Bonus Question

Draw a free hand sketch for a cross section in the shaft of question 2. The sketch should show in details the axial fixation of the pulleys and bearings.

COLLEGE OF ENGINEERING & TECHNOLOGY

Department:  Mechanical Engineering (Mechatronics)

Lecturer:  Dr. Mostafa Rostom

Course Title: Machine Design II			Course Code:  ME 357

Exam:  12th Exam					Marks:   20

Date:  6/5/2012 					Time: 1:20 hour

- The figure shows a shaft with two concentrated load. Estimate the shaft’s critical speed without the loads, the first critical speed due to the loads, and the critical speed of the combination.

A helical gear has a 30-tooth. The gear has a left-helix angle of 15°, a normal pressure angle of 20°, and a normal module is 4.0 mm. the gear width is 40 mm. Find:

  - The transverse module and transverse pressure angle

  - The normal circular pitch

  - The pitch, base, addendum, and dedendum diameters of gear

The figure is a schematic drawing of a countershaft that supports two V-belt pulleys. The belt tension on the loose side of pulley A is 15 percent of the tension on the tight side. The shaft is machined from steel with Su=550 MPa, Sy=400 MPa and Se=210 MPa. The shaft runs at 50C. The shaft has a fatigue stress-concentration factors kf=1.66 and kfs=1.63. If the fatigue factor of safety is 1.7, determine the minimum allowable diameter of the shaft at pulley A. The countershaft runs at 1200 rev/min and the bearings are to have a life of 60 kh at a combined reliability of 0.99. Select equal diameter Self Align Ball bearings for use at O and E. the bearing bore range is from 30 to 55 mm.

Bonus Question

Draw a free hand sketch for a cross section in the shaft of question 3. The sketch should show in details the axial fixation of the pulleys and bearings.